AMERISTAR FENCE PRODUCTS

Estate Gate® - Aluminum Entry Gate System

Construction Specification – SECTION 32 31 00

PART 1 - GENERAL

1.01 WORK INCLUDED

The contractor shall provide all labor, materials and apparatuses necessary for installation of the industrial ornamental steel fence system defined herein at (specify project site).

1.02    RELATED WORK

Section ___  ___  -  Earthwork

Section ___  ___  - Concrete

1.03 SYSTEM DESCRIPTION

The manufacturer shall supply a total ornamental gate system of the Ameristar® Estate Gate® (specify Classic™, Majestic™, Genesis™ or Warrior™) design.  The gate system shall include all components (i.e., gate modules, hardware, accessories and posts) required.

1.04 QUALITY ASSURANCE

The contractor shall provide laborers and supervisors who are thoroughly familiar with the type of construction involved and materials and techniques specified.

1.05 REFERENCES

- ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy Coated (Galvannealed) by the Hot-Dip Process.

- ASTM B117 - Practice for Operating Salt-Spray (Fog) Apparatus.

- ASTM B221 – Aluminum and Aluminum Alloy Extruded Bars, Shapes and Tubes

- ASTM D523 - Test Method for Specular Gloss.

- ASTM D714 - Test Method for Evaluating Degree of Blistering in Paint.

- ASTM D822 - Practice for Conducting Tests on Paint and Related Coatings and Materials using Filtered Open-Flame Carbon-Arc Light and Water Exposure Apparatus.

- ASTM D1654 - Test Method for Evaluation of Painted or Coated Specimens Subjected to Corrosive Environments.

- ASTM D2244 - Test Method for Calculation of Color Differences from Instrumentally Measured Color Coordinates.

- ASTM D2794 - Test Method for Resistance of Organic Coatings to the Effects of Rapid Deformation (Impact).

- ASTM D3359 - Test Method for Measuring Adhesion by Tape Test.

1.06 SUBMITTAL

The manufacturer's submittal package shall be provided prior to installation.

1.07 PRODUCT HANDLING AND STORAGE

Upon receipt at the job site, all materials shall be checked to ensure that no damages occurred during shipping or handling.  Materials shall be stored in such a manner to ensure proper ventilation and drainage, and to protect against damage, weather, vandalism and theft.

PART 2 - MATERIALS

2.01	MANUFACTURER

The modular gate system shall conform to Ameristar Estate (specify Classic, Majestic, Genesis, or Warrior) style, (specify 5-Rail or 5-Rail with Rings) design, (specify Single Swing or Double Swing) entry configuration, (specify standard catalog dimension) opening, (specify standard catalog) height, manufactured by Ameristar Fence Products Inc. in Tulsa, Oklahoma.

2.02	MATERIAL

A.  The materials used for gate (pickets, rails & gate ends) shall be manufactured from ASTM B221 aluminum (designation 6063-T-6) with yield strength of 25,000 PSI, a tensile strength of 30,000 PSI and a standard mill finish.

B.  Steel material for gateposts, shall be galvanized prior to forming in accordance with the requirements of ASTM A653/A653M, with minimum yield strength of 45,000 psi (310 MPa).  The steel shall be hot-dip galvanized to meet the requirements of ASTM A653/A653M with a minimum zinc coating weight of 0.90 oz/ft2 (276 g/m2), Coating Designation G-90.

B.  Material for pickets shall be 3/4” square x .125 wall aluminum; rails shall be 1” x 2” x .250” wall aluminum, gate ends and intermediates shall be 2” square x .250” wall aluminum.  Gate posts shall be per manufactures recommendation.

2.03 FABRICATION

A.  Pickets, rails and gate ends shall be precut to specified lengths, and then joined via welding at each picket to rail connection and rail to gate end connection.

B.  The manufactured framework shall be subjected to the Ameristar thermal stratification coating process (high-temperature, in-line, multi-stage, and multi-layer) including, as a minimum, a six-stage pretreatment/wash and an electrostatic spray application of a polyester finish.  The topcoat shall be a “no-mar” TGIC polyester powder coat finish with a minimum thickness of 2 mils (0.0508mm).  The color shall be (specify Black, Bronze, White, or Desert Sand).  The stratification-coated framework shall be capable of meeting the performance requirements for each quality characteristic shown in Table 1.

PART 3 - EXECUTION

3.01 PREPARATION

All new installation shall be laid out by the contractor in accordance with the construction plans.

3.02 GATE INSTALLATION

Gateposts shall be set in concrete footers having a minimum depth of 36” (Note:  In some cases, local restrictions of freezing weather conditions may require a greater depth).  The “Earthwork” and “Concrete” sections of this specification shall govern material requirements for the concrete footer.  Gateposts setting by other methods such as plated posts or grouted core-drilled footers are permissible only if shown by engineering analysis to be sufficient in strength for the intended application.

3.03 GATE INSTALLATION MAINTENANCE

When cutting/drilling gate adhere to the following steps to seal the exposed surfaces; 1) Remove all metal shavings from cut area.  2) Apply primer to thoroughly cover cut edge and/or drilled hole; let dry.  3) Apply 2 coats of custom finish paint matching fence color.  Failure to seal exposed surfaces per steps 1-3 above will negate warranty.  Ameristar spray cans or paint pens shall be used to prime and finish exposed surfaces; it is recommended that paint pens be used to prevent overspray.  Use of non-Ameristar parts or components will negate the manufactures’ warranty.

3.04 GATE INSTALLATION

Gate posts shall be spaced according to the manufacturers’ gate drawings, dependent on standard out-to-out gate leaf dimensions and gate hardware selected.  Type and quantity of gate hinges shall be based on the application; weight, height, and number of gate cycles.  The manufacturers’ gate drawings shall identify the necessary gate hardware required for the application.  Gate hardware shall be provided by the manufacture of the gate and shall be installed per manufacturer’s recommendations.

3.05 CLEANING

The contractor shall clean the jobsite of excess materials; post-hole excavations shall be scattered uniformly away from posts.

| Table 1 – Coating Performance Requirements |  |  |
| --- | --- | --- |
| Quality Characteristics | ASTM Test Method | Performance Requirements |
| Adhesion | D3359 – Method B | Adhesion (Retention of Coating) over 90% of test area (Tape and knife test). |
| Corrosion Resistance | B117 & D1654 | Corrosion Resistance over 1,000 hours (Scribed per D1654; failure mode is accumulation of 1/8” coating loss from scribe or medium #8 blisters). |
| Impact Resistance | D2794 | Impact Resistance over 60 inch lb. (Forward impact using 0.625” ball). |
| Weathering Resistance | D822 D2244, D523 (60˚ Method) | Weathering Resistance over 1,000 hours (Failure mode is 60% loss of gloss or color variance of more than 3 delta-E color units). |