# Ameristar Perimeter Security USA Inc.

Atlas-S Post & Cable – M30/P1 Engineered Swaged Cable Barrier System

Construction Specification 32 30 00.11

## PART 1 – GENERAL

### 1.01    WORK INCLUDED

The contractor shall provide all labor, materials, and appurtenances necessary for the installation of the anti-ram cable barrier system defined herein at (specify project site).

### 1.02    RELATED WORK

Section ___ ___ – Earthwork
Section ___ ___ – Concrete

### 1.03    SYSTEM DESCRIPTION

The manufacturer shall supply a complete anti-ram cable barrier system, specifically Ameristar’s Atlas-S Post & Cable – M30/P1 Engineered Swaged Cable Barrier System, which has been tested and certified as M30/P1 under ASTM F2656. The system shall include all necessary components (i.e., cables, supports, posts, and hardware).

### 1.04    QUALITY ASSURANCE

The contractor shall provide laborers and supervisors who are thoroughly familiar with the type of construction involved, as well as the materials and techniques specified.

### 1.05    REFERENCES

ASTM A36/A36M – Standard Specification for Carbon Structural Steel.

ASTM A123 - Standard Specification for Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products.

ASTM B117 - Practice for Operating Salt-Spray (Fog) Apparatus.

ASTM D523 - Test Method for Specular Gloss.

ASTM D714 - Test Method for Evaluating Degree of Blistering in Paint.

ASTM D822 - Practice for Conducting Tests on Paint and Related Coatings and Materials using Filtered Open-Flame Carbon-Arc Light and Water Exposure Apparatus.

ASTM D1654 - Test Method for Evaluation of Painted or Coated Specimens Subjected to Corrosive Environments.

ASTM D2244 - Test Method for Calculation of Color Differences from Instrumentally Measured Color Coordinates.

ASTM D2794 - Test Method for Resistance of Organic Coatings to the Effects of Rapid Deformation (Impact).

ASTM D3359 - Test Method for Measuring Adhesion by Tape Test.

ASTM F2656 – Standard Test Method for Vehicle Crash Testing of Perimeter Barriers

Federal Specification RR-W-410E / Wire Rope and Strand.

### 1.06    SUBMITTALS

The manufacturer’s product literature and crash test documentation must be submitted before installation.

### 1.07    PRODUCT HANDLING AND STORAGE

Upon delivery, all materials shall be inspected for damage. Materials shall be stored to ensure proper ventilation and drainage, and to protect against damage, weather, vandalism, and theft.

## PART 2 – MATERIALS

### 2.01    MANUFACTURER

- The anti-ram cable barrier system must comply with the Atlas-S M30/P1 Engineered Cable Barrier System design, manufactured by Ameristar Fence Products, Inc., in Tulsa, Oklahoma.

- This system shall be M30/P1 Engineered based on test results of a similar design.  Engineering analysis shall be based upon ASTM F2656, Impact Condition Designation M30, Penetration Rating P1, with the capability of stopping a 15,000 lb vehicle traveling at speeds up to 30 mph.

- The entire anti-ram cable barrier system, and all associated accessories, fittings, and fasteners shall be obtained from a single source.

### 2.02    MATERIALS

- Steel material for cable-supporting framework (i.e., posts) shall be galvanized before forming and shall conform to the requirements of ASTM A123, with a minimum yield strength of 45,000 psi (310 MPa). The steel shall be hot-dip galvanized to meet the requirements of ASTM A653/A653M, with a minimum zinc coating weight of 0.90 oz/ft² (276 g/m²), Coating Designation G-90.

- The cable material shall be an Independent Wire-Rope Core (IWRC) wire rope conforming to Federal Specification RR-W-410E, 6 x 36 Warrington Seale, preformed, right regular lay, medium lubrication, Extra Improved Plow Steel (EIPS), with a breaking strength of 159,800 pounds (79.9 tons). The cable diameter shall be 1.25 inches (excluding any PVC coating).

### 2.03    FABRICATION

- Post-weldment shall be fabricated to the specified design and lengths. System posts should be pre-drilled to accept system cables and top caps.  Cables shall be terminated inside the Terminal Posts. All structural post supports shall be located within the post and foundation and shall not have any external structural support.

- The cable assemblies shall be manufactured after field measurements are provided for the terminal post spacing for a custom fit. Cable assemblies include swaged threaded studs and nuts, along with pipe sleeves, for use with line posts.

## PART 3 – EXECUTION

### PREPARATION

- The purchaser shall stake the barrier line at intervals not exceeding 500 ft or line of sight.

- The purchaser shall identify underground utilities, USC&G benchmarks, property monuments, and other underground structures.

- Before installing the anti-ram cable barrier system, the purchaser shall perform all necessary site clearing and grading. Adequate clearance is required on both sides of the cable barrier line.

- Soil strength shall be equivalent to soil strength recorded during the ASTM F2656 certification test, or the footer size and depth must be adjusted accordingly based on professional engineering analysis.

### 3.02    INSTALLATION

The anti-ram barrier shall be installed by Ameristar’s tested M30/P1 system instructions. Posts, rails, braces, cabling, and fasteners shall be installed following the manufacturer’s installation instructions and drawings. The “Earthwork” and “Concrete” sections of this specification shall govern material requirements for the concrete footer unless otherwise specified by the product drawings or installation instructions.

3.03   FENCE INSTALLATION MAINTENANCE

If cutting/drilling of system components is required or if product coating has been compromised, adhere to the following steps to seal the exposed steel surfaces: 1) Remove all metal shavings from the cut area.  2) Apply zinc-rich primer to cover the cut edge and/or drilled hole thoroughly; let dry.  3) Apply 2 coats of custom finish paint matching the fence color.  Failure to seal exposed surfaces per steps 1-3 above will negate the warranty.  Spray cans or paint pens shall be used to prime and finish exposed surfaces; it is recommended that paint pens be used to prevent overspray.

### 3.03    CLEANING

The contractor shall thoroughly clean the jobsite, removing all debris resulting from the installation.